Logbook changes:

The FCC’s interpretation of their regulations is that the printout from the EAS endec is not sufficient.  EAS events must be entered into the station log.  While the printouts should still be stapled to the daily transmitter log, you should extract the following minimum data from the printout and copy it onto the log form.

- Time.  The EAS clock is not synched with the station clock.  Use the time from the printout, so it matches the log.  Neat, huh?

- EAS code.  This is a three-letter code for the kind of event.  Typical codes might be:
- RWT  Required weekly test
- RMT  Required monthly test
- SVS  Severe thunderstorm watch
- SVR  Severe thunderstorm warning
- FFW  Flash flood warning
- TOR  Tornado warning.

- Who we received it from.  Mostly
- KLAQ  Local Primary
- KTSM  Local Secondary
- KLYT  State Secondary (New Mexico)
- NWS  National Weather Service

Here’s how to log EAS events:

| EAS Log | (NOTE: Attach all EAS ENDEC printouts to this log form. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Time: | EAS Code: | Rec’d/ Sent? | Rec’d From: | By: | Notes/Message Summary: |
| From the printout | From the printout |  | From the printout |  |  |
| 1112 | RMT | R | KLAQ | jmc |  |
| 1113 | RMT | S |  | jmc |  |
| 1114 | RMT | R | KTSM | jmc | (duplicate - no retransmission) |
| 0915 | RWT | R | KLAQ | jmc |  |
| 1414 | RWT | R | KTSM | jmc |  |
| 1306 | RWT | S |  | jmc |  |
| 1755 | SVR | R | NWS | jmc | SC Dona Ana; W El Paso |
| 1756 | “ | “ | KLAQ | jmc | fwd NWS |
| 1757 | “ | “ | KTSM | jmc | “ |
| 1758 | “ | S |  | jmc | “ |